IMMEDIATE FIRST AID IN CASES OF ACCIDENTS WITH RADIOACTIVE MATERIALS

1.	AVOID SPREADING CONTAMINATION

Summon assistance, including an RPS, if the spillage is substantial.

Summon First aider.

Wear gloves to prevent contamination of your hands.

2.	Radioactive material

In the EYES: Irrigate with eye irrigator if available, alternatively use tap water.

In the MOUTH: Wash out with water.

On the HANDS: 		(i)	Wash thoroughly with soap and water, dry and 							monitor, pay particular attention to nails.

- If contamination persists brush area lightly with 	soap and water, avoid damaging the skin.

- If contamination persists try gentle exfoliating hand scrubs or chelating agent specific to radionuclide; avoid skin damage.

On the FACE:		(i)	Use damp swabs with soap and water.

- Ensure liquid does not touch lips or enter the 	eyes.

On other SKIN:  Treat as for hands but be careful to avoid skin damage.

Notify RPS and/or Head of Department at once if contamination persists

3.	If a wound is sustained or skin broken irrigate injury liberally with tap water.  	Encourage bleeding to a reasonable extent.  Apply first aid dressing.  Inform RPS 	and seek further advice.

4.	Record details in online incident report with the help of your DSO (AsessNet).

…………………………………….

Radiation Protection Supervisor				Tel.:

Safety Office 						Tel.: 01223 333301

IR051 (rev 1)